EX 1.2 Analysis and Measurement of High-Frequency Network Parameters using MATLAB

Objectives

i) To determine the Scattering Parameters (S-Parameters) of a two-port

microwave network by analyzing the incident and reflected wave characteristics at theinput and output ports under specified operating conditions.

ii) To analyze the variation of return loss with frequency by evaluatingthe

reflection characteristics of a microwave network over a specified frequency range.

iii) To simulate the performance of a microwave network using MATLABand

analyze its insertion loss characteristics over a specified operating range.

iv) To simulate the behavior of a high-frequency network using MATLABand

analyze its performance under different operating frequencies..

Objective 1 : Determination of Scattering Parameters (S-Parameters) of a Two-Port Microwave Network

Procedure

1. Open MATLAB and initialize the RF Toolbox.

2. Create a two-port microwave network model.

3. Define the frequency range from 1 GHz to 10 GHz.

4. Configure the network ports with characteristic impedance of 50 Ω.

5. Perform S-parameter simulation.

6. Measure S₁₁, S₁₂, S₂₁, and S₂₂ values.

7. Plot the S-parameter response over the frequency range.

8. Record the obtained results.

MATLAB Program

% Determination of Scattering Parameters (S-Parameters)

% of a Two-Port Microwave Network using MATLAB

clc;

clear;

close all;

% Experimental Data for Three Cases

% Incident Wave Amplitudes

a1 = [10 15 20]; % Port-1 Incident Wave a2 = [0 0 0]; % Port-2 Matched Load

% Reflected and Transmitted Waves

b1 = [2 3 4]; % Reflected Wave at Port-1 b2 = [8 12 16]; % Transmitted Wave at Port-2

% S-Parameter Calculations

S11 = b1 ./ a1; % Input Reflection Coefficient S21 = b2 ./ a1; % Forward Transmission Coefficient

% Assume reverse measurements

a2_rev = [10 15 20];

b2_rev = [1 1.5 2];

b1_rev = [7 10 13];

S22 = b2_rev ./ a2_rev; % Output Reflection Coefficient S12 = b1_rev ./ a2_rev; % Reverse Transmission Coefficient

% Display Results

disp('Scattering Parameters for Three Cases'); fprintf('\nCase\tS11\t\tS12\t\tS21\t\tS22\n');

for i = 1:length(a1)

fprintf('%d\t%.3f\t\t%.3f\t\t%.3f\t\t%.3f\n',... i,S11(i),S12(i),S21(i),S22(i));

end

% S-Parameter Matrix for each case

for i = 1:length(a1)

S = [S11(i) S12(i);

S21(i) S22(i)];

fprintf('\nS-Parameter Matrix for Case %d:\n',i);

disp(S);

end

% Magnitude in dB

S11_dB = 20*log10(abs(S11));

S12_dB = 20*log10(abs(S12));

S21_dB = 20*log10(abs(S21));

S22_dB = 20*log10(abs(S22));

% Plotting

figure;

plot(1:3,S11_dB,'-o','LineWidth',2);

hold on;

plot(1:3,S21_dB,'-s','LineWidth',2);

plot(1:3,S12_dB,'-^','LineWidth',2);

plot(1:3,S22_dB,'-d','LineWidth',2);

grid on;

xlabel('Case Number');

ylabel('Magnitude (dB)');

title('S-Parameter Characteristics of Two-Port Microwave Network'); legend('S11','S21','S12','S22');

Objective 2: To determine the variation of return loss with frequency and assess the

efficiency of power transmission in the network.

Procedure

1. Open the MATLAB RF simulation environment.

2. Configure the microwave network.

3. Select the desired frequency range.

4. Perform frequency sweep analysis.

5. Measure reflected power at the input port.

6. Calculate return loss using S₁₁ values.

7. Plot Return Loss versus Frequency.

8. Record and analyze the results.

MATLAB Program

% MATLAB Program to Determine the Variation of Return Loss % with Frequency and Assess Power Transmission Efficiency

clc;

clear;

close all;

% Frequency Range (GHz)

frequency = [1 2 3 4 5 6 7 8 9 10];

% Measured Reflection Coefficient (|S11|)

S11 = [0.45 0.40 0.35 0.30 0.25 0.20 0.18 0.15 0.12 0.10];

% Measured Transmission Coefficient (|S21|)

S21 = [0.60 0.65 0.70 0.75 0.80 0.85 0.88 0.90 0.93 0.95];

% Return Loss Calculation (dB)

ReturnLoss = -20 * log10(abs(S11));

% Power Transmission Efficiency (%)

Efficiency = (abs(S21).^2) * 100;

% Display Results

fprintf('Frequency\tReturn Loss\tEfficiency\n'); fprintf('(GHz)\t\t(dB)\t\t(%%)\n');

for i = 1:length(frequency)

fprintf('%.1f\t\t%.2f\t\t%.2f\n', ...

frequency(i), ReturnLoss(i), Efficiency(i)); end

% Plot 1: Return Loss vs Frequency figure;

plot(frequency, ReturnLoss, '-o', 'LineWidth', 2); grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Variation of Return Loss with Frequency');

% Plot 2: Efficiency vs Frequency figure;

plot(frequency, Efficiency, '-s', 'LineWidth', 2); grid on;

xlabel('Frequency (GHz)');

ylabel('Power Transmission Efficiency (%)'); title('Power Transmission Efficiency vs Frequency');

% Plot 3: Combined Characteristics figure;

yyaxis left

plot(frequency, ReturnLoss, '-o', 'LineWidth', 2); ylabel('Return Loss (dB)');

yyaxis right

plot(frequency, Efficiency, '-s', 'LineWidth', 2);

ylabel('Efficiency (%)');

xlabel('Frequency (GHz)');

title('Return Loss and Transmission Efficiency Characteristics'); grid on;

legend('Return Loss','Efficiency');

Objective 3: To simulate a microwave network using MATLAB and analyze its insertion loss

Procedure

1. Create the microwave network model in MATLAB.

2. Define the operating frequency range.

3. Perform transmission analysis.

4. Measure transmitted power at the output port.

5. Calculate insertion loss from S₂₁ values.

6. Repeat the analysis for different frequencies.

7. Plot Insertion Loss versus Frequency.

8. Record the observations.

MATLAB Program

% Simulation of a Microwave Network and Analysis of Insertion Loss % Using MATLAB

clc;

clear;

close all;

% Frequency Range (GHz)

frequency = 1:1:10;

% Transmission Coefficient (S21)

% Sample measured/simulated values

S21 = [0.95 0.92 0.89 0.85 0.82 0.78 0.74 0.70 0.67 0.63];

% Insertion Loss Calculation

% IL(dB) = -20log10(|S21|)

Insertion_Loss = -20*log10(abs(S21));

% Display Results

fprintf('Frequency (GHz)\tS21\tInsertion Loss (dB)\n');

for i = 1:length(frequency)

fprintf('%d\t\t%.2f\t%.2f\n', ...

frequency(i), S21(i), Insertion_Loss(i));

end

% Plot 1: S21 vs Frequency

figure;

plot(frequency,S21,'-o','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S21|');

title('Transmission Coefficient vs Frequency'); % Plot 2: Insertion Loss vs Frequency

figure;

plot(frequency,Insertion_Loss,'-s','LineWidth',2); grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss Characteristics of Microwave Network'); % Plot 3: Combined Analysis

figure;

yyaxis left

plot(frequency,S21,'-o','LineWidth',2); ylabel('|S21|');

yyaxis right

plot(frequency,Insertion_Loss,'-s','LineWidth',2); ylabel('Insertion Loss (dB)');

xlabel('Frequency (GHz)');

title('Microwave Network Performance Analysis'); legend('Transmission Coefficient (S21)', ... 'Insertion Loss');

grid on;

% Average Insertion Loss

avg_IL = mean(Insertion_Loss);

fprintf('\nAverage Insertion Loss = %.2f dB\n',avg_IL); % Network Performance Assessment

if avg_IL < 1

fprintf('Network Performance: Excellent\n'); elseif avg_IL < 3

fprintf('Network Performance: Good\n'); else

fprintf('Network Performance: Poor\n'); end

Objective 4: To simulate a high-frequency network using MATLAB and evaluate its

performance at different operating frequencies.

Procedure

1. Configure the microwave two-port network.

2. Select multiple operating frequencies.

3. Measure S-parameters at each frequency.

4. Determine return loss values.

5. Determine insertion loss values.

6. Compare network performance.

7. Plot performance comparison graph.

8. Record the results.

MATLAB Program

% Simulation of a High-Frequency Network Using MATLAB % Performance Evaluation at Different Operating Frequencies

clc;

clear;

close all;

% Operating Frequencies (GHz)

frequency = [1 2 4 6 8 10 12];

% Simulated Network Parameters

% Reflection Coefficient (S11)

S11 = [0.35 0.30 0.28 0.24 0.20 0.18 0.15];

% Transmission Coefficient (S21)

S21 = [0.65 0.70 0.75 0.80 0.85 0.88 0.92];

% Return Loss Calculation (dB)

ReturnLoss = -20*log10(abs(S11));

% Insertion Loss Calculation (dB)

InsertionLoss = -20*log10(abs(S21));

% VSWR Calculation

VSWR = (1 + abs(S11))./(1 - abs(S11));

% Power Transmission Efficiency (%)

Efficiency = (abs(S21).^2)*100;

% Display Results

fprintf('Freq(GHz)\tRL(dB)\t\tIL(dB)\t\tVSWR\tEfficiency(%%)\n');

for i = 1:length(frequency)

fprintf('%2d\t\t%6.2f\t\t%6.2f\t\t%5.2f\t%6.2f\n',... frequency(i),ReturnLoss(i),...

InsertionLoss(i),VSWR(i),Efficiency(i));

end

% Plot 1: Return Loss vs Frequency

figure;

plot(frequency,ReturnLoss,'-o','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Return Loss Characteristics');

% Plot 2: Insertion Loss vs Frequency

figure;

plot(frequency,InsertionLoss,'-s','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Insertion Loss Characteristics'); % Plot 3: VSWR vs Frequency

figure;

plot(frequency,VSWR,'-^','LineWidth',2); grid on;

xlabel('Frequency (GHz)');

ylabel('VSWR');

title('VSWR Characteristics');

% Plot 4: Efficiency vs Frequency

figure;

plot(frequency,Efficiency,'-d','LineWidth',2); grid on;

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)');

title('Power Transmission Efficiency'); % Combined Performance Analysis

figure;

plot(frequency,ReturnLoss,'-o','LineWidth',2); hold on;

plot(frequency,InsertionLoss,'-s','LineWidth',2); plot(frequency,Efficiency,'-^','LineWidth',2); grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude');

title('High-Frequency Network Performance');

legend('Return Loss','Insertion Loss','Efficiency');

% Average Performance

avg_RL = mean(ReturnLoss);

avg_IL = mean(InsertionLoss);

avg_Eff = mean(Efficiency);

fprintf('\nAverage Return Loss = %.2f dB\n',avg_RL);

fprintf('Average Insertion Loss = %.2f dB\n',avg_IL);

fprintf('Average Efficiency = %.2f %%\n',avg_Eff);